Sequencing and Scheduling: 45-865 – Spring 2000

Agenda for Week 2:

- Computational Complexity

- Basic Dispatching Rules

- Minimizing Total Completion Time (SPT rule)

- Minimizing Maximum Lateness (EDD rule)

Computational Complexity

- Algorithm

- Input data size

- Time complexity function

Polynomial vs. Exponential Computing Time

| Size (n) : Function | 10 | 20 | 30 | 40 | 50 |
| --- | --- | --- | --- | --- | --- |
| n | 0.00001 sec | 0.00002 sec | 0.00003 sec | 0.00004 sec | 0.00005 sec |
| n2 | 0.0001 sec | 0.0004 sec | 0.0009 sec | 0.0016 sec | 0.0025 sec |
| n3 | 0.001 sec | 0.008 sec | 0.027 sec | 0.064 sec | 0.125 sec |
| 2n | 0.001 sec | 1.0 sec | 17.9 min | 12.7 days | 35.7 years |

Effects of Improved Computer Technology -

Size of Largest Problem Solvable in 1 Hour:

| Function | With Present Computer | With Computer 100 Times Faster | With Computer 1000 Times Faster |
| --- | --- | --- | --- |
| n | N1 | 100N1 | 1000N1 |
| n2 | N2 | 10N2 | 31.6N2 |
| n3 | N3 | 4.64N3 | 10N3 |
| 2n | N4 | N4 + 6.64 | N4 + 9.97 |

- Polynomial time algorithm vs. Exponential time algorithm

- Exact (Optimal) Solution  Algorithm vs. Heuristic Algorithm

- Hard Problem vs. Easy Problem

- NP-hard Problems

It is very unlikely that a polynomial time algorithm exists for an  NP-hard problem

Basic Dispatching Rules

- Which job to pick next?

- A dispatching rule assigns a priority on jobs based on

Job Attributes		Machine Attributes		Current Time

- Static vs. Dynamic Rules

- Local vs. Global Rules

|  | Rule | Data | Objectives |
| --- | --- | --- | --- |
| Rules dependent on release dates | ERD | rj | Minimize variation in waiting times |
| and due dates | EDD | dj | Maximum lateness |
|  | MS | dj | Maximum lateness |
| Rules dependent on processing | LPT | pj | Load balancing over parallel machines |
| times | SPT | pj | Sum of completion times, WIP |
|  | WSPT | pj, wj | Weighted sum of completion times, WIP |
|  | CP | pj, precedences | Makespan |
|  | LNS | pj, precedences | Makespan |
| Others | SST | sjk | Makespan and throughput |
|  | LFJ | Mj | Makespan and throughput |
|  | SQNO | - | Machine utilization |
|  | SIRO | - | Ease of implementation |

Example 1:

Single-machine problem with 4 jobs with the following input data:

| Jobs | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- |
| pj | 6 | 8 | 5 | 3 |
| rj | 2 | 0 | 0 | 6 |
| dj | 16 | 18 | 15 | 21 |
| wj | 3 | 4 | 5 | 2 |

a.  Minimize sum of completion times, apply SPT

5

15

10

25

20

30

Sequence:

Total Completion time:

b.  Minimize sum of weighted completion times, apply WSPT

5

15

10

25

20

30

Sequence:

Total Weighted Completion time:

c.  Minimize maximum lateness, apply EDD

5

15

10

25

20

30

Sequence:

Maximum lateness:

d.  Minimize maximum lateness, apply MS

5

15

10

25

20

30

Sequence:

Maximum lateness:

e.  Minimize variation in waiting times, apply ERD

5

15

10

25

20

30

Sequence:

Waiting time1:

5

15

10

25

20

30

Waiting time2:

Waiting time3:			Waiting time4:

Example 2:  Consider the single-machine problem of Example 1 with the following sequence-dependent setup times:

| Jobs | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- |
| S0j | 1 | 3 | 2 | 3 |
| S1j | - | 4 | 2 | 3 |
| S2j | 3 | - | 6 | 4 |
| S3j | 2 | 3 | - | 2 |
| S4j | 3 | 1 | 5 | - |

Minimize average setup time, apply SST

5

15

10

25

20

30

Sequence:

Total setup time:

Composite Dispatching Rules

- More complex and/or multiple objectives

- Scaling parameters determine the contribution of each basic rule to the final priority

Apparent Tardiness Cost (ATC) Heuristic:

- Problem: Single-machine, n jobs available at time 0,

minimize sum of the weighted tardiness (jwjTj)  (NP-hard)

- Combine WSPT and MS

- Every time the machine becomes free, a ranking index (priority) is computed for each remaining job

Ij(t)=wjpjexp−max0,(dj−pj−t)Kp

K: scaling parameter (look-ahead parameter)

p: average of processing times

- ATC reduces to its basic rules in extreme cases:

| K | Overdue Jobs | Rule |
| --- | --- | --- |
| very large | YES/NO | WSPT |
| very small | NO | MS |
| very small | YES | WSPT (applied to overdue jobs only) |

- Determining the value of K:

- Due date tightness factor:   τ=1−dCmax

0

1

tight

loose

τ

- Due date range factor:   R=dmax−dminCmax

Example 3: Single-machine, total weighted tardiness problem with 4 jobs

| Jobs | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- |
| Pj | 13 | 9 | 13 | 10 |
| Dj | 12 | 37 | 21 | 22 |
| Wj | 2 | 4 | 2 | 5 |

- Calculate the due date tightness and due date range factors.

d= (12+37+21+22)/4 = 23

Cmax= 13+9+13+10 = 45

Due date tightness: 	τ=1−2345=0.49

Due date range: R=(22−12)45=0.22

- Apply the ATC rule with K= 5.

p= (13+9+13+10)/4=11.25

At t=0:      I1(0) = 213exp(−max(12−13−0,0)5∗11.25)=213=0.1538

I2(0) = 49exp(−max(37−9−0,0)5∗11.25)=0.2702

See the table for the rest of the calculations.

| Time | Job (j) | 1 | 2 | 3 | 4 |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | Ij(0) | 0.1538 | 0.2702 | 0.1335 | 0.4039 | Pick 4 |
| 10 | Ij(10) | 0.1538 | 0.3227 | 0.1538 |  | Pick 2 |
| 19 | Ij(19) | 0.1538 |  | 0.1538 |  | Pick 1 |
| 32 |  |  |  |  |  | Pick 3 |

C4=10, C2=19, C1=32, C3=45  and  T4= 0, T2= 0, T1= 20, T3=24

Weighted tardiness = 2*20 + 2*24 = 88

- Apply the ATC rule with K= 1.

p=11.25

At t=0:

I1(0) = 213exp(−max(12−13−0,0)1∗11.25)=213=0.1538

I2(0) = 49exp(−max(37−9−0,0)1∗11.25)=0.0369

I3(0) =

I4(0) =

Fill in the table with the rest of the calculations.

| Time | Job (j) | 1 | 2 | 3 | 4 |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | Ij(0) | 0.1538 | 0.0369 |  |  | Pick |

Weighted tardiness =

c.  Apply the ATC rule with K= 1.

p=11.25

At t=0:

I1(0) = 213exp(−max(12−13−0,0)1∗11.25)=213=0.1538

I2(0) = 49exp(−max(37−9−0,0)1∗11.25)=0.0369

See the table for the rest of the calculations.

| Time | Job (j) | 1 | 2 | 3 | 4 |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | Ij(0) | 0.1538 | 0.0369 | 0.0755 | 0.1720 | Pick 4 |
| 10 | Ij(10) | 0.1538 | 0.0897 | 0.1538 |  | Pick 1 |
| 23 | Ij(23) |  | 0.2849 | 0.1538 |  | Pick 2 |
| 32 |  |  |  |  |  | Pick 3 |

Sequence: 4-1-2-3

4

3

2

1

10

40

20

50

30

C4=10,	C1=23,	 	C2=32,	 C3=45

T4= 0, 	T1= 23-12=11,   T2= 0, 	 T3=45-21=24

Weighted tardiness = 2*11 + 2*24 = 70

OPTIMAL DISPATCHING RULES

Minimizing Total Weighted Completion Time - WSPT:

Problem: Single-machine,  Min j wjCj

WSPT: Order jobs in decreasing order of wj/pj

WSPT Rule is optimal for minimizing total weighted completion time

- Why?

- Suppose a schedule S, which is not sequenced by WSPT, is optimal

- wjpj<wkpkIn this schedule, there must be at least 2 adjacent jobs j and k such that

- Construct S' by interchanging jobs j and k

t

j

k

t+pj+pk

t

k

j

t+pj+pk

S

S'

4. Completion time of other jobs are not affected

5. Total weighted completion time of jobs j and k is

In S:  A(S)  = (t + pj) wj + ( t + pj  + pk ) wk

In S': A(S') = (t + pk) wk + ( t + pj  + pk ) wj

wjpj<wkpk

6.   A(S) - A(S') = pj wk -  pk wj  > 0 since

7. S can not be optimal

Note on Generalizations:

Hardness of variations of the single-machine, total weighted completion time problem

| jobs have different release times | Scheduler is allowed to preempt | Precedence constraints exist | jobs have equal weights | NP-hard |
| --- | --- | --- | --- | --- |
| NO | NO | YES | NO | YES |
| YES | YES | NO | NO | YES |
| YES | NO | NO | YES | YES |
| YES | YES | NO | YES | NO |
| NO | NO | NO | NO | NO |

Minimizing Maximum Lateness:

Problem: Single-machine,  Min Lmax=max(L1,...,Ln)

EDD: Order jobs in increasing order of due dates

EDD Rule is optimal for minimizing maximum lateness

- Why?

1. Suppose a schedule S, which is not sequenced by EDD, is optimal

2. In this schedule, there must be at least 2 adjacent jobs j and k such that dj > dk

3. Construct S' by interchanging jobs j and k

t

j

k

t+pj+pk

t

k

j

t+pj+pk

S

S'

4. Completion time of other jobs are not affected

5.  In S: 	Lj(S)  = t + pj - dj

Lk(S)  = t + pj  + pk  - dk

In S': 	Lk(S')  = t + pk  - dk

Lj(S')  = t + pj  + pk  - dj

6.  Lk(S) > Lk(S') and Lk(S) > Lj(S') since dj > dk

Thus, Lk(S) > max{Lk(S'), Lj(S')}

Lmax(S) >= Lmax(S')

7. Starting with S, you can obtain an EDD schedule by pairwise  interchanges such that maximum lateness is not increased

Note on Generalizations:

- If jobs are released at different points in time, the problem is  NP-hard