Sequencing and Scheduling: 45-865 – Spring 2000

Agenda for Week 3:

- LEKIN Scheduling System

- Branch and Bound and Integer Programming

- Beam Search

Branch and Bound

- General purpose method to solve discrete optimization problems

- Takes exponential time in the worst-case

- Useful to solve small instances of hard problems

- Can be applied to all scheduling problems

- Need to specify:

- A lower bounding scheme

- Branching rule

- Search rule (subproblem selection rule)

- Overview:

For a minimization problem, at a given branch and bound tree (b&b) node i:

- Obtain a lower bound, lbi

- (Optional) Obtain a candidate solution and an upper bound ubi

- If (ubi  < GLOBAL UB) then set ubi  as the GLOBAL UB

- If (lbi  >= GLOBAL UB) then prune the node and go to 6.

- Branch into subproblems (create child nodes)

- Pick the next active subproblem.

If none exist, stop. Else, go to 1.

Example 1

Problem: Single machine, n jobs arriving at different times, preemptions not allowed, minimize maximum lateness (Lmax)

- A b&b node corresponds to a partial schedule

- At level k of the b&b tree, the first k jobs in the schedule have been fixed

- Branching

Create a child node of a node at level k-1 by fixing each remaining job at kth position EXCEPT:

If a job c satisfies rc≥minl∈J(max(t,rl)+pl),

do not create a child for that job

- Lower bounding:

Schedule remaining jobs according to the preemptive EDD rule:

Every time the machine is freed or a new job is released,

pick the uncompleted job with minimum due date

Numeric Example:

| Jobs | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- |
| pj | 4 | 2 | 6 | 5 |
| dj | 8 | 12 | 11 | 10 |
| rj | 0 | 1 | 3 | 5 |

0

Level 0

1,

3,

2,

4,

lb= 5

Level 1

lb= 7

prune

prune

1,3,

1,2,

Level 2

lb= 6

lb= 5

Level 3

1,3,4,2

ub= 5

candidate

Lower bound at node (1,-) is obtained by applying preemptive EDD to jobs 2, 3 and 4 with starting time 4

At t=4, 	available jobs: 2, 3,		pick  3

At t=5, 	available jobs: 2, 3,4		pick  4

At t=10, 	available jobs: 2, 3,		pick  3

At t=15, 	available jobs: 2, 		pick  2

3

1

3

4

2

4

5

100

15

17

L1 = -4,  L2 = 5,  L3 = 4,  L4 = 0,  	Lmax = 5

Exercise: Calculate the lower bound at node (2,-)

Preemptive EDD at node (1,3,-) gives a non-preemptive schedule 1, 3, 4, 2 with Lmax = 5 and it is an optimal solution to the problem

Example 2

Problem: Single machine, minimize total weighted tardiness

- Schedules are constructed starting from the end (backwards)

- At level k of the b&b tree, the last k jobs in the schedule have been fixed, n-k jobs remain

- Branching: Create a child node of a node at level k-1 by fixing each remaining job at kth position from the end

- Lower bounding:

- Relax the problem by allowing to split the processing of a job to different time intervals

- Decompose each job j into pj jobs of unit processing time

- Define xjk = 1, if one unit of job j is processed in time interval [k-1, k]     and 0 otherwise

- Constraints:

k=1Cmaxxjk=pj   	 	 j=1,...,n

j=1nxjk=1 		k=1,...,Cmax

- Define cost coefficients cjk

cjk=0, for k≤dj and  cjk=wj, for k>dj

- Then, k=1Cmaxcjkxjk≤wjTj

- This is a “transportation problem” and can be solved in polynomial time to obtain a lower bound

Some of the partial solutions can be eliminated based on this observation:

If for two jobs j and k,  dj≤dk,  pj≤pk,  and wj≥wk, then there exists an optimal solution in which j appears before k

Numeric Example:

| Jobs | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- |
| wj | 4 | 5 | 3 | 5 |
| pj | 12 | 8 | 15 | 9 |
| dj | 16 | 26 | 25 | 27 |

Compare jobs 2 and 4: 	2 must come before 4

Compare jobs 1 and 3: 	1 must come before 3

Only jobs 3 and 4 can be last

0

,4

,3

lb=67

lb=112

,4,3

1,2,4,3

ub=67

Calculation of the lower bound at node (-,4) :

- Solve a transportation problem with jobs 1, 2, 3 and costs below

c1k=0,	k=1,...,16		c1k=4,	k=17,...,35

c2k=0,	k=1,...,26		c2k=5,	k=27,...,35

c3k=0,	k=1,...,25		c3k=3,	k=26,...,35

Supply		Jobs			Time		Demand

12		  1			  1		      1

8		  2			  2		      1

15		  3			  .		      .

.		      .

35		      1

Solution of the transportation problem:

1

3

3

2

12

190

26

35

LB = 3*9 + w4T4 = 27 + 85 = 112

- After the exploration of all active b&b nodes, it can be verified that

1, 2, 3, 4 is the best overall schedule

Integer Programming Formulations

- A linear program (LP):

linear objective and constraints,

continuous variables, infinite solution space

Min c1x1+c2x2+...+cnxn

st

a11x1+a12x2+...+a1nxn≤b1

a21x1+a22x2+...+a2nxn≤b2

.

.

.

am1x1+am2x2+...+amnxn≤bm

xj≥0	for j=1,...,n

where x1,x2,...,xn are variables

- An integer program (IP): LP with integer variables

- A mixed integer program (MIP): LP with a subset of variables integer

- An LP can be solved in polynomial time, no polynomial time algorithm is known for IP and MIP

- Branch and bound is a basic method to solve IPs and MIPs

Example

Problem: Single machine, jobs have precedence constraints, minimize total weighted completion time  (NP-hard)

- Define xjk = 1, if job j precedes job k in the sequence

xjk = 0, otherwise

- Then,	 	Cj=k=1npkxkj+pj

- Objective function:

j=1nwjCj=j=1nk=1nwjpkxkj+j=1nwjpj

minj=1nk=1nwjpkxkj+j=1nwjpj

st

xjk+xkj=1		j=1,...,n	and k=j+1,...,n

xkj+xlk+xjl≥1	j,k,l=1,...,nand j,k,l different

xjk∈0,1			j,k=1,...,n	k≠j

Precedence constraints:

Aj : Set of jobs that must precede job j

Add  xjk=1  	for all  k and  j∈Ak

Numeric Example:

| Jobs | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- |
| wj | 1 | 3 | 2 | 1 |
| pj | 6 | 8 | 12 | 9 |

Precedence Relations:

1

3

A1 and A2 are empty, A3={1,2}  A4={2}

2

4

C1=8x21+12x31+9x41+6

C2=6x12+12x32+9x42+8

C3=6x13+8x23+9x43+12

C4=6x14+8x24+12x34+9

12x13+16x23+18x43+6x14+8x24+12x3463+min8x21+12x31+9x41+18x12+36x32+27x42

st

x12+x21=1		x23+x32=1

x13+x31=1		x24+x42=1

x14+x41=1		x34+x43=1

x21+x32+x13≥1

x21+x42+x14≥1

x31+x23+x12≥1

x31+x43+x14≥1

.

.

.

x34+x23+x42≥1

xjk∈0,1			j,k=1,...,4	k≠j

x13=1, 	x23=1, 	x24=1

We can simplify the formulation as follows:

- If x13=1, then 	x31=0

- If x23=1,  then 	x32=0

- If x24=1, then 	x42=0

Eliminate x13,x31,x23,x32,x24,x42 and all valid constraints

99+min8x21+9x41+18x12+18x43+6x14+12x34

st

x12+x21=1

x14+x41=1

x34+x43=1

x21+x14≥1

x43+x14≥1

x12,x21,x14,x41,x34,x43∈0,1

The LP relaxation of this IP has an integral solution:

x14=1,	x21=1,	x34=1 with objective value 26+99=125

This solution is optimal to the original IP.

Sequence 2-1-3-4 minimizes total weighted completion time

3

2

1

4

8

14

260

35

Beam Search

- Adaptation of  branch and bound – a subset of nodes are evaluated

- At any level,

- only the promising nodes are kept for further branching

- the remaining nodes are pruned off permanently

- Since a large part of  the tree is pruned off

- running time is polynomial in input size

- It is a heuristic method

- At a level, only the best  promising nodes are kept

- Which nodes to keep?

- Estimate the potential of each node roughly

- Select  nodes for further evaluation

- Estimate the potential of the  nodes more accurately

- Select   nodes to keep

- Search parameters

- Beam width: 

- Filter width: 

- Computation time and quality of solutions depend on ,   and the node potential estimations

Level 1

Beam width = 2

Filter width = 3

Level 2

Level 3

Selected nodes

Nodes selected for further evaluation

Filtered nodes